# Activity 2 Teacher Guide: Sensors, Actuators, and Conditions

## Chapter Overview

In this chapter, students turn the GoGo Board into a game controller. They start by exploring sensors and actuators with no code at all — just the LCD screen — then write their first program in code.gogoboard.org, add decision-making with the If block, and finally build controllers that play real computer games: first a one-key vibration controller for Chrome’s Dinosaur Game, then a multi-direction GoGo Touch controller made from everyday conductive materials. The chapter payoff (playing a real game with a sensor) arrives in Activity 2.4 — the demo hook at the start of Activity 2.1 keeps students motivated through the earlier setup activities.

Focus: Sensors, actuators, and conditional logic through game controllers

Audience: Grades 7-9 (ages 12-15) | Prerequisites: Chapter 1

Total time: 4-5 hours across 6 sessions

### Learning Objectives

Students will be able to:

- Explain that sensors turn real-world events into numbers and actuators create actions

- Write and run programs in code.gogoboard.org using the Download → Run workflow

- Use the If block to make programs respond to sensor values

- Create conditions using color zones and thresholds

- Use the GoGo Board as a USB keyboard to control computer games

- Set up the GoGo Touch module and build a creative controller with conductive materials

### Chapter Structure

| Session | Activity | Time | Setup |
| --- | --- | --- | --- |
| 1 | 2.1 Exploring Sensors and Actuators | 35 min | GoGo Boards, sensor kits (no computers) |
| 2 | 2.2 Our First Program | 30 min | Add computers, USB cables |
| 3 | 2.3 Making Decisions with the If Block | 50 min | Button sensor, brightness sensor, LED |
| 4 | 2.4 Building a Game Controller | 60 min | Vibration sensor, cardboard, tape |
| 5 | 2.5 GoGo Touch Controller | 60 min | GoGo Touch, alligator clips, conductive materials |
| 6 | Assessment: Tilt Controller | 45-60 min | Computers, games that use arrow keys |

### Materials Per Group

- 1 GoGo Board with USB cable (data-capable)

- 1 computer

- Sensors: button, brightness, vibration

- Actuators: LED, DC motor with wheel

- 1 GoGo Touch module with Add-ons cable (Session 5)

- Alligator clips and conductive materials — fruits, aluminum foil, play-doh (Session 5)

- Cardboard and tape (Session 4)

- Student workbook

## Before the Chapter: Preparation

Unlike Chapter 1, no robot configuration is needed — everything in this chapter connects over USB. There is no WiFi requirement.

### Webapp Change

Starting with Activity 2.2, students switch from pgc.gogoboard.org (Chapter 1) to code.gogoboard.org — the main tool for programming the GoGo Board from here on. Connection is wired: USB cable, press the connect button in the webapp, choose “Wired (USB)”, select “GoGo Board 7”.

### Prepare a Demo Controller

Build one working game controller before Session 1 — the vibration sensor + Dinosaur Game setup from Activity 2.4 takes about 10 minutes to prepare. Showing it for 30 seconds at the start of Activity 2.1 gives students a concrete picture of what they are working toward. No explanation needed — just let them see it.

### Preparation Checklist

Equipment:

- Sort sensors and actuators into per-group kits (button, brightness, vibration, LED, DC motor)

- Check that USB cables support data transfer (some cables are charge-only)

- Test one GoGo Board with code.gogoboard.org: connect, press the bell symbol on the “Test your board” card, listen for the beep

- Check GoGo Touch modules and Add-ons cables (Session 5)

- Gather alligator clips and conductive materials — fruits, aluminum foil, play-doh, water cups (Session 5)

- Gather cardboard pieces (palm-sized or larger) and tape (Session 4)

Games (Sessions 4-6):

- Test that game links open on the school network: d.gogoboard.org/dino, d.gogoboard.org/ch2-pacman, d.gogoboard.org/games

- Build the demo controller (vibration sensor + Dinosaur Game)

## Activity 2.1: Exploring Sensors and Actuators

Time: 35 minutes | Setup: GoGo Boards, sensor kits, no computers | Grouping: Pairs or up to 4

Key concepts:

- Sensors turn real-world events into numbers

- Actuators let programs create actions in the real world

Facilitation flow:

- Demo Hook (2 min): Show the finished game controller — tap the cardboard, the dinosaur jumps. “By the end of this chapter, you will build this.” No further explanation

- Navigating the LCD (5 min): Walk through the joystick navigation once as a class before students try on their own — up/down switches screens, left/right browses within a screen, press to select

- Exploring External Sensors (10 min): Students plug in the button sensor, watch the bar graph on the Input screen, then try the brightness sensor

- Exploring Built-in Sensors (5 min): Students browse the Integrated Sensors screen — accelerometer, lux, temperature

- Exploring Actuators (10 min): Students turn a motor and an LED on and off from the Outputs screen

- Reflection (3 min): One sensor and one actuator from everyday life?

Key message: “Sensors turn the real world into numbers. Actuators turn commands into actions. We observe sensor values first — sensors do not always behave the way we expect.”

The button sensor gives values near 0 when NOT pressed and jumps to 1023 when pressed. Some students expect the opposite — this is a good teaching moment: observe first, then build.

The built-in sensors show continuous values, not just on/off. Point this out — it sets up the need for thresholds in Activity 2.4.

Common issues:

| Issue | Response |
| --- | --- |
| Students lost in LCD menus | Press up/down to return to a known screen; the screens loop around |
| Bar graph does not move | Check the sensor is plugged into an Input port (not Output) and the plug is fully seated |

Differentiation:

- Easier: Focus on the button sensor and one actuator only

- More challenging: Record actual values (idle vs. active) for every sensor on the board

## Activity 2.2: Our First Program

Time: 30 minutes | Setup: Computers, USB cables, DC motor | Grouping: Pairs or up to 4

Key concepts:

- Code gives precise control over timing and sequence

- The Download → Run workflow sends the program to the board, then runs it

Facilitation flow:

- Why Code? (3 min): Yesterday we turned the motor on from the LCD menu. But can the menu say “on for exactly 2 seconds, off for 1, on for 2, then beep”? Code can

- Connect to code.gogoboard.org (10 min): Demo on the projector — open the webapp, connect USB, choose “Wired (USB)”, press the bell symbol on the “Test your board” card to hear the beep

- Write and Run (15 min): Students build the motor on-off-on-beep program, press Download, then Run. Then they experiment: change wait times, try a short-long-short pattern

- Reflection (2 min): When is code more useful than the LCD menu?

Key message: “Code gives us precise control over timing and sequence. The LCD menu can turn things on and off, but code lets us say exactly what happens and when.”

The program itself is simple on purpose — the focus is on the environment (finding blocks, Download, Run), not the code.

Common issues:

| Issue | Response |
| --- | --- |
| Cannot connect | Check the USB cable supports data transfer. Refresh the webapp and try again |
| Nothing happens after Download | Press Run — Download only sends the program. Check the 3 purple running LEDs above the LCD |
| Motor does not spin | Check the motor is on output port 1 and the program selects port 1 |

Differentiation:

- Easier: Build the program exactly as shown, then change one wait time

- More challenging: Add more steps — a rhythm of beeps, or two motors on different ports

## Activity 2.3: Making Decisions with the If Block

Time: 50 minutes | Setup: Computers, button sensor, brightness sensor, LED | Grouping: Pairs or up to 4

Key concepts:

- Conditions are true or false — the If block runs actions only when the condition is true

- Color zones (Green / Yellow / Red) turn sensor values into simple yes/no conditions

Facilitation flow:

- Introduce Conditions (10 min): So far, programs run the same way every time. The If block lets them decide. Connect to Activity 2.1: the colored bar graphs on the LCD are the same color zones code uses — Green = low (0-350), Yellow = medium (351-700), Red = high (701-1023)

- Build If → LED (20 min): Students build “If sensor 1 is Red then LED on” inside a forever loop, test it, notice the LED never turns off, then add the else section to turn it off

- Experiments (15 min): Three variations — button → buzzer (action swap), automatic nightlight with two If blocks (sensor swap), button → motor with the “not” block (actuator swap)

- Reflection (5 min): How is the If block useful? A real-world condition?

Key message: “Programs can make decisions. This is what makes them different from a fixed sequence.”

Some students will forget the forever loop and wonder why the code only works once. This is a common and productive mistake — let them discover it, then ask: “How often does your program check the sensor?”

The experiment order matters: the action swap builds confidence, the nightlight is a relatable real-world application, and the motor experiment introduces the “not” block.

Common issues:

| Issue | Response |
| --- | --- |
| Code works only once | The forever loop is missing — the program checked the sensor one time and stopped |
| LED never turns off | The else section is missing (this is expected at Step 3 — Step 4 fixes it) |
| Condition never true | Check the sensor port number in the block matches the port used |

Differentiation:

- Easier: Stop after the If + else program with the button sensor

- More challenging: Nightlight that also beeps, or a three-zone response (Green / Yellow / Red each do something different)

## Activity 2.4: Building a Game Controller

Time: 60 minutes | Setup: Computers, vibration sensor, cardboard, tape | Grouping: Pairs or up to 4

Key concepts:

- The GoGo Board can act as a USB keyboard — sending a key press is just another output, like turning on an LED

- A threshold is a boundary value we choose ourselves — more precise than color zones

Facilitation flow:

- The Chain (5 min): Sensor → If block → keyboard key → game reacts. When the GoGo Board is connected via USB, it can send key presses like a keyboard

- Build the Pad (10 min): Tape the vibration sensor to cardboard, flip it face-down on the desk, tap and observe values on the LCD

- Choose a Threshold (10 min): Color zones give three fixed zones; a threshold is a value we pick ourselves. Use the halfway point between idle and activated values (e.g., idle 0, tap 600 → threshold 300)

- Code and Test (15 min): “If vibration > threshold then send Space” plus a short wait. Test in Notepad or keyboard-test.space first, then open d.gogoboard.org/dino and play

- Try Built-in Sensors (10 min): Accelerometer and lux sensor as controllers — blocks are in the Sensing category

- Reflection (5 min): Describe the sensor-to-game chain; what makes a good controller?

Key message: “The GoGo Board can send keyboard keys to a computer. To the computer, our sensor is now a keyboard — any program that listens to keys can be controlled by our sensor.”

The wait block after the If block matters: without it, a single tap sends the Space key many times. If students see the dinosaur jump repeatedly from one tap, ask them what the wait time is.

Encourage at least one group to try the accelerometer and share their experience — it is the sensor used in the chapter assessment. Testing in a text editor before the game separates “does the key arrive?” from “is the game hard?”.

Common issues:

| Issue | Response |
| --- | --- |
| Dinosaur does not jump | Check the program is running (3 purple LEDs above the LCD). Check the USB connection |
| Dinosaur jumps too many times | Increase the wait time |
| Response is too slow | Decrease the wait time |
| Sensor barely reacts | Tap at a spot offset from the sensor — don’t press directly on it |

Differentiation:

- Easier: Use the example threshold from the workbook and adjust from there

- More challenging: Try a different one-key game (Flappy Bird, Geometry Dash — d.gogoboard.org/games), or place the pad on the floor and stomp

## Activity 2.5: GoGo Touch Controller

Time: 60 minutes | Setup: Computers, GoGo Touch, alligator clips, conductive materials | Grouping: Pairs or up to 4

Key concepts:

- Extensions add new capabilities to code.gogoboard.org

- The GoGo Touch senses touch directly — yes/no, no threshold needed

- Conductive materials can extend the pads into creative controllers

Facilitation flow:

- Meet the GoGo Touch (5 min): 4 directional pads, A and B buttons. Connects to Add-ons port 1 (the black port). Same capacitive touch technology as a phone screen

- Mission 1 — Pac-Man (20 min): Load the GoGo Touch extension, build the setup block + forever loop + 4 If blocks (one per direction), test each pad, then play at d.gogoboard.org/ch2-pacman

- Mission 2 — Creative Controller (30 min): Students choose a game, plan which keys it needs, and extend the pads to conductive materials with alligator clips — fruits, foil, play-doh, water

- Share (5 min): Groups try each other’s controllers

Key message: “Extensions add new capabilities. And notice: the ‘Key touched’ block gives a simple yes or no — no threshold needed. Different sensors give us different kinds of answers.”

Key message (Mission 2): “Technology can be creative and playful. The physical design matters as much as the code.”

The setup block calibrates the pads at startup — no fingers on the pads (or connected materials) while it runs, and all materials must be connected BEFORE running the program. Most “broken” controllers are one of these two problems.

The most common Mission 2 problem is a missing ground connection. Remind students: pad + ground = circuit. The signal flows from the pad through the material, through the body, back to ground. Non-conductive materials (dry wood, plastic, paper) will not work — a good teaching moment about conductivity. Pencil graphite works but needs hard pressing and thick lines.

Common issues:

| Issue | Response |
| --- | --- |
| Pad does not respond | Stop the program, take all fingers off the pads, run again |
| No pads respond at all | Check the cable is in Add-ons port 1 (black), not port 2 (purple). If the cable was replugged while running, restart the board |
| Key sends too many times | Add a short wait after each tap key block |
| Materials not registering | Check the ground connection. Is the material conductive? Try holding a ground wire in the other hand |
| Pad triggers on its own | Materials were connected after the code started — stop and re-run |

Differentiation:

- Easier: Complete Mission 1, then extend just one pad to one material

- More challenging: Combine with Activity 2.4 — add a vibration sensor for a jump action (Space) alongside the GoGo Touch directions

## Session 6 Assessment: Tilt Controller

Time: 45-60 minutes | Format: Practical demonstration + reflection

Note: This is an additional activity designed specifically for assessment, so it does not appear in the student workbook. Present the challenge using the Session 6 slides, and copy or display the student task sheet at the end of this section. Students write their answers on the sheet or on paper.

### The Challenge

Use the GoGo Board’s built-in accelerometer to build a tilt-based game controller.

How it works:

- Tilt the board left → send the left arrow key

- Tilt the board right → send the right arrow key

- Optional: tilt forward / backward → send the up / down arrow keys

Facilitation flow:

- Introduction (5-10 min): Read the challenge, demo tilting a board while watching the accelerometer value on the LCD. If the “Try Built-in Sensors” section in Activity 2.4 was skipped, do this demo carefully — this is students’ first meeting with the accelerometer

- Student Work (30-35 min): Observe tilt values, choose thresholds, write the If blocks, pick a game with arrow keys, test and adjust

- Presentations (5-10 min): 3-5 groups demonstrate and explain their code

- Reflection (5 min): Students answer the 2 reflection questions on the task sheet

Getting started guidance for students: The accelerometer block is in the Sensing category. Find the value when the board is flat (the middle point) — tilting one way makes the value go higher, the other way lower. Choose thresholds on each side, just like the vibration sensor in Activity 2.4. Remember the forever loop.

Game ideas: Pac-Man, Snake, Tetris, Breakout, Space Invaders, Donkey Kong — or d.gogoboard.org/games

While students work, observe who tests systematically — observing values before choosing thresholds — and who guesses. This informs the rubric scoring below.

### Assessment Rubric

Practical (60%):

| Criterion | Not yet (1-2) | Developing (3-4) | Proficient (5-6) |
| --- | --- | --- | --- |
| Working controller | No working If block with the accelerometer | 2-direction controller works (left/right) | 4-direction controller works |
| Thresholds | Copied without testing | Chosen from observed values | Chosen and refined after testing |
| Code explanation | Cannot explain | Explains partially | Explains If blocks and thresholds clearly |

Presentation (40%):

| Criterion | Not yet (1-2) | Developing (3-4) | Proficient (5-6) |
| --- | --- | --- | --- |
| Depth of explanation | Surface-level | Some examples | Specific, clear |
| Communication | Unclear | Adequate | Clear, organized |

### Student Task Sheet (copy or display)

This page presents the assessment activity in the student-facing voice — copy it for each student or display it on the projector.

The Challenge: Tilt Controller

Use the GoGo Board’s built-in accelerometer to build a game controller that works by tilting the board.

Tilting the GoGo Board to control a game

How it works:

- Tilt the board left → send the left arrow key

- Tilt the board right → send the right arrow key

- Optional: tilt forward / backward → send the up / down arrow keys

Getting started: The accelerometer block is in the Sensing category. Tilt the board left and right while watching the value on the LCD. Find the value when the board is flat — this is the middle point. Use these observations to choose threshold values, just like the vibration sensor in Activity 2.4. Remember to use a forever loop.

Step 1: Write the program using If blocks and the accelerometer

How many If blocks are needed? . . . . . . .

Step 2: Choose a game that uses arrow keys

Game ideas: Pac-Man, Snake, Tetris, Breakout, Space Invaders, Donkey Kong — or find more at d.gogoboard.org/games

Game we chose: . . . . . . . . . . . . . . . . . . . . . . . . . .

Step 3: Test and adjust

What accelerometer value triggers a “tilt left”? . . . . . . .

What value triggers a “tilt right”? . . . . . . .

Checklist:

- Write the program with If blocks for left and right tilt

- Test and find the right threshold values

- Play the game using tilt controls

- Optional: add up and down tilt for 4-direction control

- Explain the program to the class

Game: . . . . . . . | Directions: 2 / 4 | Best score: . . . . . . .

Reflection:

1. How is the tilt controller different from the button and sensor controllers we built before?

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

2. Why does the threshold value matter for the accelerometer?

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

## Troubleshooting

Connection:

| Problem | Solution |
| --- | --- |
| Board not detected | Make sure the USB cable supports data transfer (some cables are charge-only). Try a different USB port |
| Webapp cannot connect | Refresh the webapp, reconnect USB, try again |
| No beep from “Test your board” | Reconnect and select “GoGo Board 7” again in the popup |

Game controllers:

| Problem | Solution |
| --- | --- |
| Game does not react | Check the program is running (3 purple LEDs above the LCD). Check USB. Click on the game window so it receives key presses |
| Key sends too many times | Increase the wait time after the If / tap key block |
| Response too slow | Decrease the wait time |

GoGo Touch:

| Problem | Solution |
| --- | --- |
| Pad does not respond | Stop the program, remove fingers from all pads, run again |
| No pads respond | Check the cable is in Add-ons port 1 (black), not port 2 (purple). Restart the board if the cable was replugged while running |
| Materials not registering | Check the ground connection and that the material is conductive. Try holding a ground wire in the other hand |
| Pad triggers on its own | Materials were connected after the program started — stop and re-run |

## Vocabulary Reference

| Term | Definition |
| --- | --- |
| Sensor | A device that detects something in the real world (light, touch, vibration) and turns it into a number |
| Actuator | A device that creates a physical action — a motor spins, an LED lights up |
| Condition | A check that is either true or false (e.g., “Is the sensor in the Green zone?”) |
| If block | A block that runs an action only when a condition is true |
| Forever loop | A loop block that repeats its contents endlessly — used to keep checking sensors |
| Color zone | Sensor values divided into Green (0-350), Yellow (351-700), and Red (701-1023) — simple conditions without choosing a number |
| Threshold | A specific sensor value chosen as a boundary for a condition — unlike color zones, we pick the value ourselves |
| HID keyboard | A mode where the GoGo Board sends keyboard key presses to the computer via USB |
| Accelerometer | A built-in sensor that detects tilt, shake, and orientation of the board |
| Extension | An add-on loaded in code.gogoboard.org to support extra hardware like the GoGo Touch |
| GoGo Touch | A capacitive touch module with 4 directional pads and 2 buttons, connected to Add-ons port 1 (black) |
| Capacitive touch | Detecting touch through electrical signals — touching a pad (or a connected conductive material) completes a circuit through our body |
| Calibrate | To measure and adjust a sensor so it works correctly — the GoGo Touch calibrates its pads at startup |